DLD  Lab-04

Karnaugh Map

NATIONAL UNIVERSTIY OF COMPUTER AND EMERGING SCIENCES, FAST- Peshawar Campus

Department Of Computer Science

EL1005 – Digital Logic Design-Lab

SEMESTER SPRING 2026

Contents

1.	Objectives:	2

2.	Outcomes :	2

3.	Equipment Required:	2

1-	Simplification of 3 variable Boolean function using Kmap	2

1.	Min term(product)	3

2.	Max term(SUM)	4

3.	Min Term & Max term	4

4.	Sum of Product (Min Term)	5

5.	Product of Sum (Max term)	6

6.	Karnaugh Map	8

7.	Lab Task	13

# Objectives:

3/4 variable Boolean function simplification using K-map in SOP (sum of products)  and POS (product of sums ) forms

Implementation of simplified expressions using AND, OR and NOT gates

Implementation of simplified expressions using NAND and NOR gates

# Outcomes:

Students should be able to

Simplify, implement and verify 3/4 variable Boolean expressions in SOP and POS forms,

and using only NAND and only NOR gates.

# Equipment Required:

- DEV-2765E Trainer Board/ Multisim 14.2 /Logic.ly

- 7404 hex NOT (Inverter) gate IC

- 7408 quad 2-input AND gate IC

- 7432 quad 2-input OR gate IC

- 7400 quad 2-input NAND gate ICs

- 7402 quad 2-input NOR gate ICs

- Multisim / Logicly

# Simplification of 3 variable Boolean function using Kmap

In  the lab  there are 2 ACs and a door . There is a monitoring room as well . If  any of the AC is on , AND  the  door is open ( i.e logic one for the door ) , then  an output signal (i.e  logic 1 ) is issued to the monitoring room , otherwise output remains zero .

Make a truth table corresponding to the above scenario, use A1 and A2 for ACs and D for door and S for output signal , simplify the Boolean function into

(a) sum-of-products form         and        (b) product-of-sums form

# Min term(product)

Minterm is product of boolean variables either in normal form or complemented form. In Minterm, we look for the functions where the output results in “1”

# Max term(SUM)

Maxterm is sum of boolean variables either in normal form or complemented form. In Maxterm we look for function where the output results in “0”

# Min Term & Max term

# Sum of Product (Min Term)

The SOP (Sum of Product) is the methods for deducing a particular logic function. Conversely, SOP produces a logical expression comprised of the OR of the multiple AND terms

A Sum-Of-Products Boolean expression is exactly a set of Boolean terms added together, each term being a product of a combination of Boolean variables. To get the Sum of Products form from a truth table, OR together all of the min terms which has a value of 1.

# Product of Sum (Max term)

The POS (Product of Sum) is the methods for deducing a particular logic function. Conversely, POS produces a logical expression comprised of the AND of the multiple OR terms

A Product of Sum Boolean expression is exactly a set of Boolean terms product together, each term being a added of a combination of Boolean variables. To get the Product of Sum form from a truth table, product together all of the max terms which has a value of 0.

# Karnaugh Map

# Lab Task