DLD  Lab-07

Magnitude Comparator

NATIONAL UNIVERSTIY OF COMPUTER AND EMERGING SCIENCES, FAST- Peshawar Campus

Department Of Computer Science

EL 1005– Digital Logic Design-Lab

SEMESTER SPRING 2026

Contents

1.	Objectives:	2

2.	Digital Comparator	2

3.	Magnitude Comparator	3

4.	1-Bit Magnitude Comparator	3

5.	2-Bit Magnitude Comparator	6

6.	How to design a 4–bit comparator?	9

7.	Applications of Comparators	11

8.	Code converters	12

9.	BCD, Excess-3 code	12

10.	Binary, BCD, Excess-3 code	13

11.	Students Task	15

# Objectives:

Magnitude Comparator

- Design and implement the circuitry for 1-bit magnitude comparator.

- Design and implement the circuitry for 2-bit magnitude comparator.

- Design and implement the circuitry for 4-bit magnitude comparator

- Truth Table

- K-Map

- Equation

- Circuit Diagram

- Implementation

Code Converters

Design and implement the circuitry for a BCD-to-Excess 3 Code Converter.

# Digital Comparator

- It is a combinational (circuit without memory) logic circuit.

- Digital Comparator is used to compare the value of two binary digits.

- There are two types of digital comparator

(i) Identity Comparator

(ii) Magnitude Comparator.

- IDENTITY COMPARATOR:

  - This comparator has only one output terminal for when A=B, either A=B=1 (High) or A=B=0 (Low)

- MAGNITUDE COMPARATOR:

  - This Comparator has three output  terminals namely A>B, A=B, A<B. Depending on the result of  comparison, one of these output will be high (1)

- Block Diagram of Magnitude Comparator is shown below in Fig. 1

# Magnitude Comparator

# 1-Bit Magnitude Comparator

- This magnitude comparator has two inputs A and B and three outputs:

- A<B, A=B and A>B.

- This magnitude comparator compares the two numbers of single  bits.

- Truth Table of 1-Bit Comparator

# 2-Bit Magnitude Comparator

- A	comparator which is used to compare two binary numbers each of two

- bits is called a 2-bit magnitude comparator.

- Fig. 2 shows the block diagram of 2-Bit magnitude comparator.

- It has four inputs and three outputs.

- Inputs are A0 ,A1,B0 and B1 and Outputs are Y1, Y2 and Y3

TRUTH TABLE

# How to design a 4–bit comparator?

The truth table for a 4-bit comparator would have 4^4 = 256 rows. So we will do things a bit differently here. We will compare each bit of the two 4-bit numbers, and based on that comparison and the weight of their positions, we will draft a truth table.

- In a 4-bit comparator the condition of A>B can be possible in the following four cases:

If A3 = 1 and B3 = 0

If A3 = B3 and A2 = 1 and B2 = 0

If A3 = B3, A2 = B2 and A1 = 1 and B1 = 0

If A3 = B3, A2 = B2, A1 = B1 and A0 = 1 and B0 = 0

- Similarly the condition for A<B can be possible in the following four cases:

If A3 = 0 and B3 = 1

If A3 = B3 and A2 = 0 and B2 = 1

If A3 = B3, A2 = B2 and A1 = 0 and B1 = 1

If A3 = B3, A2 = B2, A1 = B1 and A0 = 0 and B0 = 1

- The condition of A=B is possible only when all the individual bits of one number exactly coincide with corresponding bits of another number.

- A=B: (A3 Ex-Nor B3) (A2 Ex-Nor 82) (Al Ex-Nor BI) (AO Ex-Nor BO)

# Applications of Comparators

- These are used in the address decoding circuitry in computers and microprocessor based devices to select a specific input/output device for the storage of data.

- These are used in control applications in which the binary numbers representing physical variables such as temperature, position, etc. are compared with a reference value. Then the outputs from the comparator are used to drive the actuators so as to make the physical variables closest to the set or reference value.

- Process controllers

- Servo-motor control

- Used in password verification and biometric applications.

# Code converters

- The Code converter is used to convert one type of binary code to another.

- There are different types of binary codes like BCD code, gray code, excess-3 code, etc. Different codes are used for different types of digital applications.

- To get the required code from any one type of code, the simple code conversion process is done with the help of combinational circuits.

- A code converter circuit will convert coded information in one form to a different coding form.

# BCD, Excess-3 code

# Binary, BCD, Excess-3 code

# Students Task

- Implement 1 bit, 2 bit and 4 bit comparator on logic lay

- Implement Excess 3 Converter circuit on on logic ly

- Design K-map and circuit diagram for Gray Code like Excess 3