DLD  Lab-10

7-Segment Display

NATIONAL UNIVERSTIY OF COMPUTER AND EMERGING SCIENCES, FAST- Peshawar Campus

Department Of Computer Science

EL1005 – Digital Logic Design-Lab

SEMESTER SPRING 2026

Contents

1.	Objectives:	2

2.	Outcomes:	2

3.	Equipment Required:	2

4.	Seven Segment LED Display:	2

5.	Two  types of  7 Segment Displays	3

6.	Working of 7-Segment Display (LED & LCD) Circuit	6

7.	7-Segment Truth Table	11

8.	7-Segment Display Decoder Circuit	15

9.	Application of BCD to Display Decoder	15

10.	IC-74LS47 Implementation	16

# Objectives:

Familiarization with operation of 7-segment display

Interfacing with BCD to Seven Segment Code converter IC

Design of BCD to 7-segment code converter circuit

# Outcomes:

Students should be able to

Interface a 7 segment display with BCD to  7 segment code converter IC

Design and implement the BCD  to 7 segment code converter using Kmaps

# Equipment Required:

- DEV-2765E Trainer Board / Multisim 14.2 /Logic.ly

- 7448 BCD to 7-segment code converter

- Common-cathode 7-segment LED (single and double digit) Display

# Seven Segment LED Display:

Decimal digits from 0 to 9 can be shown using a 7–segment LED display unit. This unit shows one decimal digit using 7 LED segments to form the numbers from 0 to 9. These 7-segments are a, b, c, d, e, f, g

7-Segment Display

A BCD-to-seven-segment decoder (7448) is a combinational circuit that converts a decimal digit in BCD to an appropriate code for the selection of segments in an indicator used to display the decimal digit in a familiar form. The seven outputs of the decoder (a, b, c, d, e, f, g) select the corresponding segments in the display, as shown in Fig.(a) . The numeric display chosen to represent the decimal digit is shown in Fig.(b) .

# Two  types of  7 Segment Displays

Different  decoder  IC  with  different  display

7447 (BCD to 7-segment Converter )  outputs are active low , it  is  used with common anode display .

7448 (BCD to 7-segment Converter )  outputs are active high , it  is  used with common cathode display  .

Pin configuration

7448-IC Pin Configuration

The pin connections of the common-cathode 7-segment single digit display are below:

Procedure

- Connect the trainer with the power supply

- Mount the BCD to 7-segment driver IC 74LS48 on the trainer board

- Connect pins of the IC according to the diagram

- Connect pin 16 to +5 V (Power Supply) and pin 8 to GND (Ground).

- Apply all combinations of inputs leaving pins 3,4,5 unconnected  and see the corresponding decimal digit on the 7-segment LED display .Show these to the instructor

Design of  BCD to seven segment code converter

Using truth tables and Karnaugh maps, design the BCD-to-seven-segment decoder using minimum number of gates.

# Working of 7-Segment Display (LED & LCD) Circuit

# 7-Segment Truth Table

for output a:

for output b:

for output b:

for output c:

for output d:

for output e:

for output f

for output g

# 7-Segment Display Decoder Circuit

# Application of BCD to Display Decoder

- This circuit can be used as a timer circuit.

- With little modification, it can be also used to display the number clock pulses.

- It can also be used with modification to display alphabet display system instead of decimal display system.

- 7-Segment display are mostly used in digital clocks, electronic meters, odometers as well as LCD application due to low current consumption.

- They are also used in various measuring instruments, digital watches and digital counter

# IC-74LS47 Implementation  in Multisim